# Project 1: Organizing and Summarizing Air Quality Data

Scope of Work

Jessica Tackett

CIVE 202: Civil Engineering Analysis II

February 3, 2026

Contents

Project Goals:	3

Project Tasks:	3

Project Deliverables:	5

References	6

Scope of Work:

Team Members: Jessica Tackett

Date submitted for review: 2/3/2026

Submitted to: Civil and Environmental Engineering Department Faculty

Project Goals:

The goal of this project is to analyze and summarize data from the AirPurple dataset to see if Nebraska’s air quality is meeting National Ambient Air Quality Standards, identifying where pollution hotspots are, how environmental factors contribute to air quality, and the potential health concerns. The data is collected from various locations across Nebraska, allowing UNMC to understand how air pollution affects human health. UNMC has requested five key analyses: (1) identifying the 5 locations in Nebraska with the highest mean and median measurements for concentrations of VOC, PM2.5, and PM 10.0, (2) dates where the maximums of the pollutants occurred and their potential causes, (3) analysis of whether humidity and temperature have an relation to air quality, (4) seeing  where and when there have been any Air Quality Index (AQI) health risks, with regard to measurements of PM 2.5 and PM 10.0, and seeing if there are any potential causes and finally, (5) whether the sensor altitude has an impact on the air quality values.

# Project Tasks:

Task #1: What are the 5 locations in Nebraska with the highest mean and median concentrations of VOC, PM 2.5 and PM 10.0?

In order to accomplish task 1, our engineering team will use python and pandas data analysis library to summarize the dataset and determine the top five locations with the highest mean and median concentration of pollutants. Since this is our first task, we must import the dataset, provided by the client, from a CSV file and then turn the data into a data frame. This will only have to be done once. The data will then be grouped by the sensor location (sensor.name) to organize the data by where the pollutants occur. Then, statistical summaries will be computed for VOC, PM 2.5, and PM 10.0 for the mean and median of the pollutants. Using a pivot table, the data can be viewed in a clean way, showing the top five areas with the highest mean and median values of pollutants. This data shows locations with higher air quality concern.

Task #2: On what days did the maximum values occur and where did this maximums occur?

- What are some potential reasons these maximums occurred on these days in these locations based on initial research?

For this task, our engineering team will group the data by date and location (sensor.name). Grouping by date and location allows the team to track pollutant levels overtime and see if there are any reasons the maximum values occurred when they did. Then a statistical summary of the max values will be coded. The data will be shown using a pivot table. This is a clean way to present data for the client. The result of this task will allow the client to review the days and locations where the maximum values and then conduct further research as to why these maximums may have occurred.

Task #3: Does humidity and temperature have a noticeable effect on air quality?

a. Please use the following categories for humidity:

i. Low humidity occurs at a relative humidity less than 50%, high humidity occurs at a relative humidity between 50 and 80%, very high humidity occurs at a relative humidity above 80%

b. Please use the following categories for temperature:

The first step our engineering team took for this task was to categorize the temperature and humidity using the ranges and categories given to the team by the client prompt. This allows the team to see any relation between humidity and temperature on air quality. Then, new columns are added to the data with our new category assignments. This will be shown using a pivot table which will organize the data in a clean way. The result of the task will show if temperature or humidity have any correlation with higher pollutant numbers.

Task #4: Have there been any Air Quality Index (AQI) health risks (unhealthy for sensitive populations) at any of the locations in the dataset for PM 2.5 and PM 10 based on the EPA’s AQI ratings?

a. When did they happen?

b. Based on exploratory research, what could have been a potential cause?

Task four involved some research over the EPA’s threshold for unhealthy air conditions for sensitive populations. With the findings, our group sorted the PM 2.5 and PM 10.0 and filtered the values on whether they exceeded the EPA and AQI’s thresholds. For PM 2.5, 35 µg/m^3 is the start of the unhealthy range for sensitive groups (“First in Air Quality”). For PM 10.0, this level is 154 µg/m^3. Then, the data was printed by date and location to show the areas and dates where the unhealthy air conditions occurred. With further research, our team can see probable causes for these unhealthy air conditions, whether it is weather, wildfire smoke, industrial emission, or traffic pollution.

Task #5 BONUS: Does the sensor altitude have an impact on the air quality values?

The first step our team conducted for this task was grouping the data by sensor altitude and then running a statistic summary for the max, min, mean, and median of the various pollutants. Next, a correlation analysis will be conducted between sensor altitude and the three pollutants to see if the altitude has any correlation with air quality. After reviewing the correlation coefficients, our team will be able to determine if sensor altitude has an impact on air quality values.

# Project Deliverables:

The deliverables and due dates for this project are summarized in the following table:

| Deliverable | Files Included | Due to Client by |
| --- | --- | --- |
| Raw Data file | AirQuality_Daily_StudentVersion.csv | 2/3/26 |
| Python Code | CIVE202_Spring_2026_JessicaTackett_Project1_PythonCode | 2/3/26 |
| Annotated Code Document | CIVE202_Spring_2026_JessicaTackett_Project1_AnnotatedCode.xlsx | 2/3/26 |
| Written Report | CIVE202_Spring_2026_JessicaTackett_Project1_WrittenReport.docx | 2/3/26 |

# References

Works Cited

“First in Air Quality.” IQAir, www.iqair.com/us/support/knowledge-base/KA-05074-US. Accessed 02 Feb. 2026.

“Particulate Matter (PM) Basics.” EPA, Environmental Protection Agency, www.epa.gov/pm-pollution/particulate-matter-pm-basics. Accessed 02 Feb. 2026.