# Project 1: Organizing and Summarizing Air Quality Data

Written Summary

Jessica Tackett

CIVE 202: Civil Engineering Analysis II

February 3, 2026

Contents

Introduction/Background:	3

Scope of Work	4

Methods:	6

Results:	7

References	10

# Introduction/Background:

UNMC has partnered with AirPurple to monitor air quality conditions across Nebraska. The individuals at UNMC do not have the proper knowledge or expertise regarding air quality, so they are asking for our engineering firm to complete the data analysis of the AirPurple dataset. Specifically, UNMC wants to know if the air quality meets the National Ambient Air Quality Standards (NAAQS). They also want to know if the areas with worse air quality are related to public health concerns. The group wants to understand how temperature, elevation, geographic location, and humidity impact the air quality to see if there is any relation between the variables and health. For this project, our engineering group will analyze the air data collected from around Nebraska and see if there are any environmental factors that contribute to these air quality values.

To further understand this analysis, the pollutants and their meanings must be known. The first pollutant is volatile organic compounds (VOC). VOCs are compounds that have a low water solubility and high vapor pressure. Many of the VOCs in the air are the result of the manufacturing of paints, pharmaceuticals, and refrigerants. This pollutant also consists of a plethora of chemicals, each having different short- and long-term health effects (“What Are Volatile Organic Compounds”). The second pollutant covered in this analysis in PM 2.5. PM stands for particulate matter. PM 2.5 are generally 2.5 micrometers or smaller, making them a fine inhalable particle. PM 10.0 is our third pollutant is generally 10 micrometers or smaller making it another inhalable particle. These particles can be made up of many different chemicals emitted from various sources. These sources include, but are not limited to, constructions sites, fields, unpaved roads, and fires. Since these particles are so small, they can be inhaled and cause serious health problems (“Particulate Matter”).

# Scope of Work

Scope of Work:

Team Members: Jessica Tackett

Date submitted for review: 2/3/2026

Submitted to: Civil and Environmental Engineering Department Faculty

Project Goals: The goal of this project is to analyze and summarize data from the AirPurple dataset to see if Nebraska’s air quality is meeting National Ambient Air Quality Standards, identifying where pollution hotspots are, how environmental factors contribute to air quality, and the potential health concerns. The data is collected from various locations across Nebraska, allowing UNMC to understand how air pollution affects human health. UNMC has requested five key analyses: (1) identifying the 5 locations in Nebraska with the highest mean and median measurements for concentrations of VOC, PM2.5, and PM 10.0, (2) dates where the maximums of the pollutants occurred and their potential causes, (3) analysis of whether humidity and temperature have an relation to air quality, (4) seeing  where and when there have been any Air Quality Index (AQI) health risks, with regard to measurements of PM 2.5 and PM 10.0, and seeing if there are any potential causes and finally, (5) whether the sensor altitude has an impact on the air quality values.

Project Tasks:

Task #1: What are the 5 locations in Nebraska with the highest mean and median concentrations of VOC, PM 2.5 and PM 10.0?

In order to accomplish task 1, our engineering team will use python and pandas data analysis library to summarize the dataset and determine the top five locations with the highest mean and median concentration of pollutants. Since this is our first task, we must import the dataset, provided by the client, from a CSV file and then turn the data into a data frame. This will only have to be done once. The data will then be grouped by the sensor location (sensor.name) to organize the data by where the pollutants occur. Then, statistical summaries will be computed for VOC, PM 2.5, and PM 10.0 for the mean and median of the pollutants. Using a pivot table, the data can be viewed in a clean way, showing the top five areas with the highest mean and median values of pollutants. This data shows locations with higher air quality concern.

Task #2: On what days did the maximum values occur and where did these maximums occur?

What are some potential reasons these maximums occurred on these days in these locations based on initial research?

For this task, our engineering team will group the data by date and location (sensor.name). Grouping by date and location allows the team to track pollutant levels overtime and see if there are any reasons the maximum values occurred when they did. Then a statistical summary of the max values will be coded. The data will be shown using a pivot table. This is a clean way to present data for the client. The result of this task will allow the client to review the days and locations where the maximum values and then conduct further research as to why these maximums may have occurred.

Task #3: Does humidity and temperature have a noticeable effect on air quality?

a. Please use the following categories for humidity:

i. Low humidity occurs at a relative humidity less than 50%, high humidity occurs at a relative humidity between 50 and 80%, very high humidity occurs at a relative humidity above 80%

The first step our engineering team took for this task was to categorize the temperature and humidity using the ranges and categories given to the team by the client prompt. This allows the team to see any relation between humidity and temperature on air quality. Then, new columns are added to the data with our new category assignments. This will be shown using a pivot table which will organize the data in a clean way. The result of the task will show if temperature or humidity have any correlation with higher pollutant numbers.

Task #4: Have there been any Air Quality Index (AQI) health risks (unhealthy for sensitive populations) at any of the locations in the dataset for PM 2.5 and PM 10 based on the EPA’s AQI ratings?

a. When did they happen?

b. Based on exploratory research, what could have been a potential cause?

Task four involved some research over the EPA’s threshold for unhealthy air conditions for sensitive populations. With the findings, our group sorted the PM 2.5 and PM 10.0 and filtered the values on whether they exceeded the EPA and AQI’s thresholds. For PM 2.5, 35 µg/m^3 is the start of the unhealthy range for sensitive groups (“First in Air Quality”). For PM 10.0, this level is 154 µg/m^3. Then, the data was printed by date and location to show the areas and dates where the unhealthy air conditions occurred. With further research, our team can see probable causes for these unhealthy air conditions, whether it is weather, wildfire smoke, industrial emission, or traffic pollution.

Task #5 BONUS: Does the sensor altitude have an impact on the air quality values?

The first step our team conducted for this task was grouping the data by sensor altitude and then running a statistic summary for the max, min, mean, and median of the various pollutants. Next, a correlation analysis will be conducted between sensor altitude and the three pollutants to see if the altitude has any correlation with air quality. After reviewing the correlation coefficients, our team will be able to determine if sensor altitude has an impact on air quality values.

Project Deliverables:

The deliverables and due dates for this project are summarized in the following table:

| Deliverable | Files Included | Due to Client by |
| --- | --- | --- |
| Raw Data file | AirQuality_Daily_StudentVersion.csv | 2/3/26 |
| Python Code | CIVE202_Spring_2026_JessicaTackett_Project1_PythonCode | 2/3/26 |
| Annotated Code Document | CIVE202_Spring_2026_JessicaTackett_Project1_AnnotatedCode.xlsx | 2/3/26 |
| Written Report | CIVE202_Spring_2026_JessicaTackett_Project1_WrittenReport.docx | 2/3/26 |

# Methods:

The data analysis done for this project was conducted using Python in Jupyter Notebook. In addition, the data analysis library our engineering team used was the pandas library. First, our team imported the air quality data from air purple to use as our data for this project. This dataset included our three pollutants (VOC, PM 2.5, PM 10.0), temperature, humidity, altitude, dates, and locations across Nebraska. In order to evaluate the sensor by location, our team grouped the data by the sensor name. By running a summary statistic, we were able to calculate the mean and median for these locations. A pivot table was created to summarize the data and allowed us to compare the results. To identify maximums, the data was grouped by date and location (sensor.name). The maximums were then extracted and sorted into another pivot table for us to analyze data in a clear way. To evaluate weather impacts on pollutants, humidity and temperature were sorted and categorized into different ranges. New columns were then created to showcase these categories, and another pivot table was made to cleanly analyze the data. To assess levels that were unhealthy for sensitive populations, the Air Quality Index (AQI) was used to filter instances where the PM 2.5 and PM 10.0 values exceeded the unhealthy for sensitive populations. Finally, a correlation analysis was conducted to see if there was a relationship between sensor altitude and air quality values. All of these analyses were done using functions such as .groupby(), .agg(), pivot_table(), a .corr().

# Results:

Task #1:

The analysis done for task one allowed our team to identify five locations in Nebraska with the highest mean and median concentrations of VOC, PM 2.5, and PM 10.0. Some of our key findings were that Broken Bow exhibited the highest mean values of PM 10.0 and PM 2.5 at values of 929.678 and 928.711. These values well exceed the healthy air quality values given by EPA’s AQI ratings. For PM 2.5, the unhealthy for sensitive populations starts at values 35, and for PM 10.0, 154. The median values of the pollutants were also high for this particular area. As for the other areas, the medians were in range for good air quality. As for VOCs, Otoe County exhibited the highest mean in this category at a value of 251.427. But Otoe County did have relatively good means for the PM values. The causes of higher particulate matter could be due to agricultural dust, industrial activities, or seasonal weather patterns. This would make sense with our findings as most of these areas are in more rural regions of Nebraska, having more dirt roads and agricultural dust. Looking at the VOC levels, these high values can be due to fuel combustion, chemical use, and urban activity.

Task #2:

For task two our team identified the days and locations where the highest values of pollutants occurred. The results of this data showed the highest concentrations of January first as well as December 31st. What these days have in common is the New Year which many people let off fireworks in celebration. The specific pollutant that is released when using fireworks is PM 2.5 and gases such as sulfur dioxide and nitrogen oxide (Marybeth). This would make sense with our findings as the PM 2.5 values on these days were incredibly high at over 2,000 for multiple locations such as Broken Bow and Richard County Courthouse. PM 10.0 and VOC values were also very high with some being in the two thousands and over 200 which could also be from fireworks.

Task #3:

From our findings we cannot see any evident evidence that there is a relationship between humidity and temperature on pollutants. Although some areas of the results start to show a slight correlation, it is not consistent with each group. When doing some further research on the Purple Air community provided by the client, our team found that the temperature and humidity data are for the sensor itself and are not reflective of the environmental conditions. Heat produced by the WiFi module causes an increase in temperature and a decrease in humidity within the housing (“What Do Purpleair”). This would make sense with our findings, as there was little correlation between temperature and humidity on pollution.

Task #4:

From the data, our team found that there are many locations and dates when there have been air quality health risks. This is when the PM 2.5 exceeds 35 and the PM 10.0 exceeds 154. From our table, we are able to see that the top events with high values occurred at the Richardson County Courthouse from 2/18/25-2/20/25 as well as 1/21/25. Broken bow also exhibited high values on 2/18/25. All of these levels are in the three thousands which far exceeds the EPAs unhealthy levels. Hazardous levels are anywhere from 400-500 suggesting that these are most likely due to a sensor error. These numbers also were measured around the same time. If this is not an error, then possible causes could be due to wildfire smoke or agricultural activity since these are in more rural areas.

Task #5:

From our correlation function, our group was able to analyze the relationship between sensor altitude and air quality values. To further understand the relationship, our team will provide the meaning of the values below. The data showed that there is not a strong correlation between altitude and pollution. The VOC relationship came out to .1246 meaning there is little to no relationship. The PM 10.0 value came out to -.058 meaning there is a very weak negative relationship. PM 2.5 was around the same value of -.058 also meaning there is a very weak negative relationship. This shows that altitude does not have a very noticeable relationship with air quality values.

1.0 = very positive relationship

0.5 = moderate positive relationship

0 = no relationship

-0.5 = moderate negative relationship,

-1.0 = very negative relationship

# References

Works Cited

Collins, Marybeth. “The Environmental Impact of Fireworks: Celebrations and Consequences.” Environment+Energy Leader, 28 Jan. 2026, environmentenergyleader.com/stories/the-environmental-impact-of-fireworks-celebrations-and-consequences,44790. Accessed 03 Feb. 2026.

“First in Air Quality.” IQAir, www.iqair.com/us/support/knowledge-base/KA-05074-US. Accessed 02 Feb. 2026.

“Get Air Quality Data Where You Live.” AirNow.Gov, www.airnow.gov/?city=false&country=false. Accessed 03 Feb. 2026.

PurpleAir. “What Do Purpleair Sensors Measure and How Do They Work?” PurpleAir Community, 3 Feb. 2023, community.purpleair.com/t/what-do-purpleair-sensors-measure-and-how-do-they-work/3499. Accessed 03 Feb. 2026.

“Particulate Matter (PM) Basics.” EPA, Environmental Protection Agency, www.epa.gov/pm-pollution/particulate-matter-pm-basics. Accessed 02 Feb. 2026.

“US EPA PM2.5 Air Quality Standards Interactive Map by PurpleAir.” PurpleAir, map.purpleair.com/air-quality-standards-us-epa-aqi?opt=%2F1%2Flp%2Fa10%2Fp604800%2FcC0#5.21/40.002/-97.008. Accessed 03 Feb. 2026.

“What Are Volatile Organic Compounds (VOCs)?” EPA, Environmental Protection Agency, www.epa.gov/indoor-air-quality-iaq/what-are-volatile-organic-compounds-vocs. Accessed 02 Feb. 2026.